EXP:2(a)

Date :15/10/26

clc;

clear;

close all;

% Experiment 2(A): Transmission-Line S-Parameters

% 50 Ohm microwave transmission line

% Input parameters

Z0 = 50;                 % Reference impedance (Ohms)

ZL = 50;                 % Load impedance (Ohms)

L = 50e-3;                % Line length (50 mm)

c = 3e8;                  % Speed of light (m/s)

f = linspace(8e9, 12e9, 501); % Frequency sweep

f0 = 10e9;                % Centre frequency (10 GHz)

% Ideal matched transmission line

beta = 2*pi*f/c;

S11 = zeros(size(f));

S22 = zeros(size(f));

S21 = exp(-1j*beta*L);

S12 = exp(-1j*beta*L);

% Convert magnitudes to dB (floor for plotting zero)

S11_dB = 20*log10(max(abs(S11), 1e-6));

S22_dB = 20*log10(max(abs(S22), 1e-6));

S21_dB = 20*log10(abs(S21));

S12_dB = 20*log10(abs(S12));

% Calculate output parameters

idx = find(f >= f0, 1);

RL = -20*log10(max(abs(S11(idx)), 1e-12));

IL = -20*log10(abs(S21(idx)));

eta = abs(S21(idx))^2 * 100;

% Display output

fprintf('--- OUTPUT PARAMETERS AT 10 GHz ---\n');

fprintf('S11 = %.4f dB (ideal: -Inf dB)\n', S11_dB(idx));

fprintf('S12 = %.4f dB\n', S12_dB(idx));

fprintf('S21 = %.4f dB\n', S21_dB(idx));

fprintf('S22 = %.4f dB (ideal: -Inf dB)\n', S22_dB(idx));

fprintf('Return Loss = Infinite (ideal match)\n');

fprintf('Insertion Loss = %.4f dB\n', IL);

fprintf('Transmission Efficiency = %.2f %%\n', eta);

% Graph 1: S-parameter magnitudes

figure;

plot(f/1e9, S11_dB, 'r', 'LineWidth', 2);

hold on;

plot(f/1e9, S12_dB, 'b', 'LineWidth', 2);

plot(f/1e9, S21_dB, 'g', 'LineWidth', 2);

plot(f/1e9, S22_dB, 'm', 'LineWidth', 2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

title('S-Parameters of 50 Ohm Transmission Line');

legend('S11','S12','S21','S22','Location','best');

ylim([-100 5]);

hold off;

% Graph 2: Return loss

figure;

plot(f/1e9, -S11_dB, 'r', 'LineWidth', 2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Return Loss (dB)');

title('Return Loss versus Frequency');

% Graph 3: Insertion loss

figure;

plot(f/1e9, -S21_dB, 'b', 'LineWidth', 2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Insertion Loss (dB)');

title('Insertion Loss versus Frequency');

% Graph 4: Transmission efficiency

figure;

plot(f/1e9, abs(S21).^2*100, 'g', 'LineWidth', 2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Transmission Efficiency (%)');

title('Transmission Efficiency versus Frequency');

ylim([0 105]);

OUTPUT: