EXP:1(a)

Date:14/10/26

clc;

clear;

close all;

a = 22.86e-3;

b = 10.16e-3;

L = 100e-3;

f = 10e9;

c = 3e8;

k0 = 2*pi*f/c;

fc = c/(2*a);

beta = (2*pi*f/c)*sqrt(1-(fc/f)^2);

fprintf('Theoretical Cutoff Frequency = %.4f GHz\n',fc/1e9);

fprintf('Propagation Constant = %.4f rad/m\n',beta);

x = linspace(0,a,100);

y = linspace(0,b,60);

[X,Y] = meshgrid(x,y);

Ey = sin(pi*X/a);

Hx = zeros(size(X));

Hy = zeros(size(X));

Hz = cos(pi*X/a);

figure(1);

imagesc(x*1000,y*1000,Ey);

set(gca,'YDir','normal');

colorbar;

xlabel('x (mm)');

ylabel('y (mm)');

title('TE10 E-field Distribution');

figure(2);

imagesc(x*1000,y*1000,Hz);

set(gca,'YDir','normal');

colorbar;

xlabel('x (mm)');

ylabel('y (mm)');

title('TE10 H-field Distribution');

figure(3);

plot(x*1000,abs(Ey(1,:)),'LineWidth',2);

xlabel('x (mm)');

ylabel('|Ey|');

title('TE10 Electric Field Variation');

grid on;

figure(4);

plot(x*1000,abs(Hz(1,:)),'LineWidth',2);

xlabel('x (mm)');

ylabel('|Hz|');

title('TE10 Magnetic Field Variation');

grid on;

fc_sim = input('Enter simulated cutoff frequency from HFSS (GHz): ');

deviation = abs(fc_sim-fc/1e9)/(fc/1e9)*100;

fprintf('Theoretical Cutoff Frequency = %.4f GHz\n',fc/1e9);

fprintf('Simulated Cutoff Frequency = %.4f GHz\n',fc_sim);

fprintf('Percentage Deviation = %.4f %%\n',deviation);

if f > fc

fprintf('TE10 mode propagates at 10 GHz.\n');

else

fprintf('TE10 mode is below cutoff.\n');

end

OUTPUT: