EXP:1(b)

Date:14/10/26

clc;

clear;

close all;

% Experiment 2(B): Optimum Frequency Analysis

% Input parameters

Z0 = 50;                 % Reference impedance (Ohms)

L = 50e-3;               % Transmission line length (50 mm)

c = 3e8;                 % Speed of light (m/s)

f = linspace(8e9,12e9,501);

fGHz = f/1e9;

f0 = 10e9;               % Expected optimum frequency

% Frequency-dependent illustrative transmission-line model

beta = 2*pi*f/c;

% Reflection coefficient with a small residual mismatch

Gamma = 0.005 + 0.2*((f-f0)/(2e9));

% S-parameters

S11 = Gamma .* exp(-1j*2*beta*L);

S22 = S11;

IL_model = 0.1 + 0.03*(fGHz-10).^2; % Illustrative loss in dB

S21 = sqrt(1-abs(S11).^2) ...

.* 10.^(-IL_model/20) .* exp(-1j*beta*L);

S12 = S21;

% S-parameter magnitudes in dB

S11_dB = 20*log10(abs(S11));

S21_dB = 20*log10(abs(S21));

S12_dB = 20*log10(abs(S12));

S22_dB = 20*log10(abs(S22));

% Find optimum frequencies

[~,idx1] = min(abs(S11));

[~,idx2] = max(abs(S21));

% Use the frequency where S11 is minimum

idx = idx1;

% Output parameters at optimum frequency

f_opt = f(idx);

RL = -20*log10(abs(S11(idx)));

IL = -20*log10(abs(S21(idx)));

eta = abs(S21(idx))^2*100;

fprintf('\n--- OPTIMUM FREQUENCY RESULTS ---\n');

fprintf('Minimum |S11| frequency = %.3f GHz\n',f(idx1)/1e9);

fprintf('Maximum |S21| frequency = %.3f GHz\n',f(idx2)/1e9);

fprintf('Optimum frequency       = %.3f GHz\n',f_opt/1e9);

fprintf('S11 at optimum          = %.4f dB\n',S11_dB(idx));

fprintf('S21 at optimum          = %.4f dB\n',S21_dB(idx));

fprintf('Return loss             = %.4f dB\n',RL);

fprintf('Insertion loss          = %.4f dB\n',IL);

fprintf('Transmission efficiency = %.2f %%\n',eta);

% Graph 1: S11 and S21

figure;

plot(fGHz,S11_dB,'r','LineWidth',2);

hold on;

plot(fGHz,S21_dB,'b','LineWidth',2);

plot(f_opt/1e9,S11_dB(idx),'ro','MarkerSize',8);

plot(f(idx2)/1e9,S21_dB(idx2),'bo','MarkerSize',8);

grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

title('S11 and S21 versus Frequency');

legend('S11','S21','Minimum S11','Maximum S21',...

'Location','best');

hold off;

% Graph 2: Return loss

figure;

plot(fGHz,-S11_dB,'r','LineWidth',2);

hold on;

plot(f_opt/1e9,RL,'ko','MarkerSize',8);

grid on;

xlabel('Frequency (GHz)');

ylabel('Return Loss (dB)');

title('Return Loss versus Frequency');

hold off;

% Graph 3: Insertion loss

figure;

plot(fGHz,-S21_dB,'b','LineWidth',2);

hold on;

plot(f(idx2)/1e9,-S21_dB(idx2),'ko','MarkerSize',8);

grid on;

xlabel('Frequency (GHz)');

ylabel('Insertion Loss (dB)');

title('Insertion Loss versus Frequency');

hold off;

% Graph 4: Transmission efficiency

figure;

plot(fGHz,abs(S21).^2*100,'g','LineWidth',2);

hold on;

plot(f(idx2)/1e9,abs(S21(idx2))^2*100,...

'ko','MarkerSize',8);

grid on;

xlabel('Frequency (GHz)');

ylabel('Transmission Efficiency (%)');

title('Transmission Efficiency versus Frequency');

hold off;

% Graph 5: All four S-parameters

figure;

plot(fGHz,S11_dB,'r','LineWidth',1.8);

hold on;

plot(fGHz,S21_dB,'b','LineWidth',1.8);

plot(fGHz,S12_dB,'g--','LineWidth',1.8);

plot(fGHz,S22_dB,'m--','LineWidth',1.8);

grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

title('Complete S-Parameter Sweep');

legend('S11','S21','S12','S22','Location','best');

hold off;

OUTPUT: